# Self-inspection Checklist – Open Sources

This form may be found useful as an “aide memoire” to RPS’s prior to the annual inspection by the Radiation Safety Unit, as well as for intermediate inspections.

| School: |  |
| --- | --- |
| RPS: |  |
| Date: |  |
| Inspection By: |  |
| Others Present |  |

General

| 1 | Are all users registered as Radiation Workers? | YES | NO |
| --- | --- | --- | --- |
|  | Have all Radiation Workers received sufficient training to enable them to work safely? | YES | NO |
| 3 | Are Local Rules available in each area? | YES | NO |
| 4 | What form of radiation is used? |  |  |

Radiation Areas

| 5 | Are all rooms containing open sources listed? | YES |  |  | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Is the information in the Local Rules correct? | YES |  |  | NO |  |  |
| 7 | Are all areas used for radiation work clearly demarcated? | YES |  |  | NO |  |  |
| 8 | Is all flooring sheet vinyl with welded seams & integral coving? | YES |  |  | NO |  |  |
| 9 | Are all floor coverings in good condition? | YES |  |  | NO |  |  |
| 10 | Are all bench surfaces smooth, hard, non-absorbent with sealed joints? | YES |  |  | NO |  |  |
| 11 | Are bench surfaces in good condition? | YES |  |  | NO |  |  |
| 12 | Are benchcote and/or plastic trays used? | YES |  |  | NO |  |  |
| 13 | Are working areas and benches monitored? | YES |  |  | NO |  |  |
| 14 | Are records of the monitoring results available? | YES |  |  | NO |  |  |

Controlled Areas

| 15 | Is there appropriate radiation signage on door? | YES |  |  | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | Is a written System of Work displayed? | YES |  |  | NO |  |  |
| 17 | Are there suitable access restrictions? | YES |  |  | NO |  |  |

Radioisotopes

| 18 | Are all radioisotopes kept in lockable stores? | YES |  |  | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | Are stores appropriately labelled? | YES |  |  | NO |  |  |
| 20 | Is there an up-to-date stock list with each store? | YES |  |  | NO |  |  |
| 21 | Do School Stock Records match contents of isotope stores? | YES |  |  | NO |  |  |
| 22 | Are all radioisotopes still in use? | YES |  |  | NO |  |  |

Disposals

| 23 | Are labelled bins provided for radioactive solid waste? | YES |  |  | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | Is there a designated disposal sink in each isotope room? | YES |  |  | NO |  |  |
| 25 | Are all disposal sinks appropriately labelled? | YES |  |  | NO |  |  |
| 26 | Are the associated waste pipes labelled? | YES |  |  | NO |  |  |
| 27 | Are sinks clean & free from excessive contamination? | YES |  |  | NO |  |  |
| 28 | Do all disposals conform to the Local Rules? | YES |  |  | NO |  |  |
| 29 | Are up to date disposal records available? | YES |  |  | NO |  |  |

Monitors

| 30 | Are suitable radiation monitor(s) available? | YES |  |  | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | Do all monitors respond to radioactivity? | YES |  |  | NO |  |  |
| 32 | Are battery checks satisfactory? | YES |  |  | NO |  |  |
| 33 | Has the performance of monitors been checked within last 12 months? | YES |  |  | NO |  |  |

Emergency Planning

| 34 | Is a suitable kit available for dealing with spillages? | YES |  |  | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | Have all potential radiation hazards been assessed? | YES |  |  | NO |  |  |
| 36 | Are contingency plans in existence for emergencies | YES |  |  | NO |  |  |

Action Required

| Auditor Signature: |  | Witnessed By: |  |
| --- | --- | --- | --- |
| Position: |  | Position: |  |

NOTES:

| 1 | Radiation Workers observed in laboratories can be checked against the Radiation Worker Register. The Radiation Safety Unit should be informed immediately of any unregistered workers. |
| --- | --- |
| 2 | Assessment of individual competency based on working practices/observed conditions in labs. |
| 3/4 | Each laboratory should have a copy of Local Rules available and visible. |
| 5/6 | Inform Radiation Safety Unit of changes |
| 7 | Signage available via Radiation Safety Unit. |
| 8-12 | Flooring & benches should be constructed, maintained and used so that they do not readily become contaminated and, should contamination occur, they can be easily decontaminated. Defects (faulty seals, cracks, bare wood exposed, etc) should be dealt with promptly. |
| 13 14 | It is vital that regular contamination monitoring is undertaken - working area monitoring and regular surveys of the entire laboratory. Records relating to monitoring must be kept - and be both readily available and interpretable |
| 16 | Non-classified Radiation Workers may only enter Controlled Area whilst following a written System of Work. This should be displayed prominently in the area. Copies of such systems of work should be sent to the Radiation Safety Unit. |
| 17 | Locked at all times when not occupied - access restricted to those directly involved in the work i.e. no spectators (even if they are themselves registered workers). |
| 18 19 | All stores should be lockable and be marked with a prominent radiation hazard sign. No food or flammables in such a store. |
| 20 21 | Stock records must be kept with each store. All items must be recorded and each individual container (‘pot’) should be clearly labelled with a radiation hazard trefoil together with the radionuclide, activity, receipt/activity date and a unique identifier such as a serial number, which links the pot to the stock record. All dispensations of radionuclides must be recorded (at that time) in the relevant stock record and should include the activity used, activity remaining and the person performing the dispensation. |
| 22 | Old stocks must not be allowed to accumulate and must be disposed of via an authorised route. The Radiation Safety Unit should be contacted for advice prior to disposal of large quantities of such waste. |
| 23 | All bins should be clearly marked and situated within the ‘radiation area’ (where this is only part of a room). Siting both normal clinical waste bins and radioactive clinical waste bins in close proximity should be avoided. Similarly, using both “black bags” (for very low level waste with no other biohazard) and “yellow bags” (for clinical radioactive waste) in the same area is not recommended. |

| 24 | A Designated Sink is not necessarily required in every radiation area - if any aqueous waste can be safely transported to a Designated Sink outside the laboratory. |
| --- | --- |
| 25 26 | Labels/stickers can be obtained from the Radiation Safety Unit |
| 27 | Sinks need to be monitored too! |
| 28 29 | All disposals must be recorded. These disposal records should be readily available |
| 30 | Suitable monitors should be readily available, preferably within the radiation area itself. It is not recommended that monitors are “shared” between labs - where this cannot be avoided there should be good (and documented) reason for such arrangements (e.g. very short term work, intermittent use, etc.) |
| 31 | Any which do not respond should be removed immediately from the radiation area. The Radiation Safety Unit should be contacted to arrange for repair/replacement - temporary ‘bridging’ loan of a monitor may be possible. Contamination Monitors are regarded as essential pieces of safety equipment in Schools using radioactivity - all costs are School ones. |
| 32 | If NO, withdraw from use until batteries replaced. Check when last used! |
| 33 | If NO contact the Radiation Safety Unit as appropriate. |
| 35 36 | The extent to which emergency planning requires consideration will vary with School. As a minimum, School personnel should be aware of whom to contact in the event of a radiation emergency. The Radiation Safety Unit should be contacted for advice if the requirements are unclear |